# Source Code — Key Excerpts

Appendix — Development of a Software Teach Pendant for a 7-DOF Robot Arm

This appendix collects the key source-code excerpts of the teach pendant, grouped into the pendant application and the ROS 2 motion-backend nodes it drives. Each excerpt is reproduced verbatim from the repository; the path and the exact source line numbers are given beneath every heading, and omitted code is shown as “# ...”.

# 1  Teach Pendant Application

The teach pendant is a standalone Qt (PyQt6) desktop application that couples to the robot only through a small set of ROS 2 topics. The excerpts below show the signature parts of each module; routine setup and boilerplate are elided with “# ...”.

## ROS Bridge — Interface Contract and Cartesian Jog

teach_pendant/pendant7dof/ros_bridge.py   ·   The single rclpy node owned by the GUI: the joint-name/limit contract, the publishers and subscribers for the contract topics, and the workspace-clamped Cartesian jog that keeps the end-effector target inside the reachable dome.   [excerpt — source lines 29–53, 62–72, 121–132, 156–166, 381–404, 427–435, 460–461]

JOINT_NAMES = [

"joint_1", "joint_2", "joint_3", "joint_4",

"joint_5", "joint_6", "joint_7",

]

# "Home" button = all joints at zero (per user request). NB this is the

# straight-out, near-singular pose, so jogging/drawing FROM it can be poorly

# conditioned — it's a parking/reset target, not the operating pose. (The

# backend still BOOTS to the conditioned elbow-up go_to_start pose

# [0,-0.4,0,1.2,0,0,0] in pendant_backend.launch.py; that is separate.)

HOME_JOINTS = [0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0]

JOG_STEP_RAD = 0.1

JOG_DURATION_S = 0.5

# Soft joint limits kept just inside the URDF hard limits (matches

# drawing_ui_node). ``None`` = continuous joint.

LIMIT_MARGIN_RAD = 0.05

JOINT_LIMITS = [

None,            # joint_1 — continuous

(-1.6, 1.6),     # joint_2

None,            # joint_3 — continuous

(-1.6, 1.6),     # joint_4

None,            # joint_5 — continuous

(-0.48, 0.26),   # joint_6 — narrow range

(-1.6, 1.6),     # joint_7

]

# ...

DRIVER_NODES = ("ik_7dof_v3", "ik_to_trajectory", "drawing_batch_planner")

# Named joint presets shown as buttons in the GUI.

PRESETS: dict[str, list[float]] = {

"Home": list(HOME_JOINTS),

"Ready": [0.0, 0.4, 0.0, -0.6, 0.0, 0.0, 0.0],

}

# Cartesian jog step (metres) and the frame targets are published in.

CART_STEP_M = 0.01

BASE_FRAME = "base_link"

# ...

def __init__(self) -> None:

super().__init__("pendant7dof_bridge")

# --- publishers ---------------------------------------------------

self.traj_pub = self.create_publisher(

JointTrajectory, "/arm_controller/joint_trajectory", 10

)

self.target_pub = self.create_publisher(PoseStamped, "/ee_target", 10)

self.strokes_pub = self.create_publisher(String, "drawing/strokes", 10)

# Publishing an empty PoseArray clears any in-flight drawing path so

# its IK output stops fighting manual commands.

self.path_pub = self.create_publisher(PoseArray, "/cartesian_path", 10)

# ...

# --- subscriptions ------------------------------------------------

self.create_subscription(JointState, "/joint_states", self._cb_state, 20)

self.create_subscription(PoseStamped, "/ee_pose", self._cb_ee, 20)

self._pen_cb = None

self.create_subscription(Point, "/pen_canvas_norm", self._cb_pen, 30)

# Watch the planner's drawing trajectory to learn its real duration.

self.create_subscription(

JointTrajectory, "/arm_controller/joint_trajectory", self._cb_traj, 10)

self.get_logger().info("pendant7dof bridge ready")

# ...

def cartesian_jog_xyz(self, dx: float, dy: float, dz: float) -> None:

"""Stream an EE target from the joystick (~15 Hz), integrating stick

deflection into an internal target that leads the actual pose. The

target is clamped into the reachable dome so it can't be pushed out of

reach; if the EE still stalls, the pose is held cleanly."""

if self._estopped or self._ee_pose is None:

return

if dx == 0.0 and dy == 0.0 and dz == 0.0:

# Stick centered / released: stop leading the target and re-anchor it

# to the actual pose so the live jog IK reaches it and DEACTIVATES.

# Otherwise the last ~1 cm lead (or an unreachable lead at a reach

# limit) keeps ik_arm_v3 perpetually active, streaming /joint_commands

# -> ik_to_trajectory -> /arm_controller/joint_trajectory at ~50 Hz,

# which then preempts any drawing trajectory ~50x/s (the twitch).

self._cart_jog_stop()

return

p = self._ee_pose.pose.position

cur = [p.x, p.y, p.z]

if self._cart_target is None:

self._cart_target = list(cur)

self._jog_moved_once = False   # fresh jog: wait for the IK to engage

step = math.sqrt(dx * dx + dy * dy + dz * dz)

d_unit = [dx / step, dy / step, dz / step]

# ...

# Build the leading target, clamped to within CART_LEAD_M of the actual,

# then projected into the reachable dome (never outside the workspace).

t = [self._cart_target[0] + dx,

self._cart_target[1] + dy,

self._cart_target[2] + dz]

lead = self.CART_LEAD_M

for i in range(3):

t[i] = min(cur[i] + lead, max(cur[i] - lead, t[i]))

t, _ = self._clamp_target(t, cur)

# ...

self._cart_target = t

self._publish_ee_target(*t)

## Main Window — Jog Dispatch and Simulation Control

teach_pendant/pendant7dof/gui/main_window.py   ·   How the GUI dispatches user input: the joint/Cartesian mode and joint-group toggles, the joystick callback that streams jog commands to the bridge, and the Simulation on/off control that launches the ROS backend.   [excerpt — source lines 1076–1095, 1799–1849]

def _selected_mode(self) -> str:

return "gazebo" if self.mode_combo.currentIndex() == 0 else "moveit"

def _toggle_backend(self) -> None:

if self.backend is None:

return

if self.backend.running:

self.backend.stop()

self._set_sim_btn(False)

self.mode_combo.setEnabled(True)

else:

mode = self._selected_mode()

args = [f"mode:={mode}"]

if mode == "gazebo":

# Gazebo already shows its own 3D view; don't also open RViz.

args.append("rviz:=false")

self.backend.start(extra_args=args)

self._set_sim_btn(True)

self.mode_combo.setEnabled(False)

# ...

def _toggle_jog_mode(self) -> None:

self.jog_mode = "cartesian" if self.jog_mode == "joint" else "joint"

self._update_jog_ui()

def _toggle_jog_group(self) -> None:

# Cycle 1–3 → 4–6 → Joint 7 → 1–3 (same button as the 1–3/4–6 switch).

self.jog_group = (self.jog_group + 1) % 3

self._update_jog_ui()

def _update_jog_ui(self) -> None:

cartesian = self.jog_mode == "cartesian"

self.mode_toggle_btn.setText("Mode: Cartesian" if cartesian else "Mode: Joint")

self.group_toggle_btn.setVisible(not cartesian)

self.joint_set_box.setVisible(not cartesian)

self.cart_set_box.setVisible(cartesian)

# Cartesian jog: lock the joystick to one axis at a time (slider feel) so

# only X or only Y moves per drag. Joint modes keep the free 2-axis knob.

self.joystick.set_axis_lock(cartesian)

if cartesian:

self.joystick.set_labels("X", "Y", "Z")

elif self.jog_group == 2:

# Joint 7 alone: only the vertical (+Y/−Y) axis drives it; the X

# axis and the twist arc carry no label, so they're hidden + idle.

self.group_toggle_btn.setText("Joint 7")

self.joystick.set_labels("", "J7", "")

else:

base = 0 if self.jog_group == 0 else 3

self.group_toggle_btn.setText("Joints 1–3" if self.jog_group == 0 else "Joints 4–6")

# Short labels on the joystick itself (J1..J6 = joint_1..joint_6).

self.joystick.set_labels(

f"J{base+1}", f"J{base+2}", f"J{base+3}"

)

def _on_joy(self, x: float, y: float, twist: float) -> None:

spd = self.jog_speed.value()

if self.jog_mode == "cartesian":

k = CART_STEP_PER_TICK * spd

# A zero tick (stick released/centered) re-anchors the jog IK so it

# goes idle instead of chasing the last leading target forever.

self.node.cartesian_jog_xyz(x * k, y * k, twist * k)

else:

if x == 0.0 and y == 0.0 and twist == 0.0:

return   # release in joint mode: nothing to command

k = JOINT_STEP_PER_TICK * spd

if self.jog_group == 2:

# Joint 7 (index 6) on the vertical axis only; X/twist ignored.

self.node.jog_joints({6: y * k})

else:

base = 0 if self.jog_group == 0 else 3

self.node.jog_joints({base: x * k, base + 1: y * k, base + 2: twist * k})

## Jog Joystick Widget

teach_pendant/pendant7dof/gui/joystick.py   ·   The custom two-axis jog knob: per-drag axis-lock (slider feel for Cartesian jog), the fixed-rate velocity-style stream to the consumer, and the final zero tick on release (paint methods omitted).   [excerpt — source lines 28–61, 258–298]

class Joystick(QWidget):

def set_labels(self, x_label: str, y_label: str, twist_label: str) -> None:

self.x_label, self.y_label, self.twist_label = x_label, y_label, twist_label

self.update()

def set_axis_lock(self, enabled: bool) -> None:

"""Constrain the knob to one axis at a time (slider-like). The chosen

axis is re-picked on each fresh grab."""

self.axis_lock = bool(enabled)

self._locked_axis = None

def __init__(self, on_jog=None, rate_hz: float = 15.0) -> None:

super().__init__()

self.on_jog = on_jog

self.setMinimumSize(220, 220)

self.x = 0.0          # -1..1

self.y = 0.0          # -1..1 (up = +)

self.twist = 0.0      # -1..1

# Axis-lock: when on, the knob commits to a single axis (whichever the

# user first pulls toward) and the other axis is held at 0 for the rest

# of the drag, so the control behaves like an X-or-Y slider. Used by

# Cartesian jog so only one Cartesian axis moves at a time.

self.axis_lock = False

self._locked_axis = None   # None | 'x' | 'y' (chosen per drag)

# Short labels for what each direction drives (set by the consumer).

self.x_label = "X"

self.y_label = "Y"

self.twist_label = "Z"

self._knob_active = False

self._ring_active = False

self._grab_angle = 0.0

self._timer = QTimer(self)

self._timer.timeout.connect(self._tick)

self._tick_ms = int(1000.0 / rate_hz)

# ...

def mouseReleaseEvent(self, _e):

self._knob_active = False

self._ring_active = False

self.x = self.y = self.twist = 0.0

self._locked_axis = None

self._timer.stop()

# Emit one final centered tick so the consumer learns the jog stopped

# (the timer is now stopped, so _tick won't fire again). The Cartesian

# bridge uses this zero tick to re-anchor and let the jog IK go idle;

# without it the IK keeps chasing the last leading target.

if self.on_jog is not None:

self.on_jog(0.0, 0.0, 0.0)

self.update()

def _update_knob(self, pos):

c, r_base, *_ = self._metrics()

dx = (pos.x() - c.x()) / r_base

dy = -(pos.y() - c.y()) / r_base

if self.axis_lock:

# Commit to the dominant axis once the knob is pulled clear of a

# small centre deadzone, then hold that axis for the rest of the

# drag so only X *or* Y ever moves (slider feel). The other axis is

# zeroed, so the knob travels along a straight line.

if self._locked_axis is None and math.hypot(dx, dy) > 0.18:

self._locked_axis = "x" if abs(dx) >= abs(dy) else "y"

if self._locked_axis == "x":

dy = 0.0

elif self._locked_axis == "y":

dx = 0.0

else:

dx = dy = 0.0   # not committed yet → stay put (no diagonal leak)

mag = math.hypot(dx, dy)

if mag > 1.0:

dx, dy = dx / mag, dy / mag

self.x, self.y = dx, dy

def _tick(self):

if self.on_jog is not None:

# Negate twist so a clockwise turn drives the positive direction;

# the grip still rotates with the finger (paint uses self.twist).

self.on_jog(self.x, self.y, -self.twist)

## Drawing Canvas Widget

teach_pendant/pendant7dof/gui/drawing_canvas.py   ·   Stroke capture on the millimetre-scaled canvas (strokes hold rather than trail the edge when the mouse leaves), the JSON drawing handed to the planner, and the live pen-tip dot that fades when the pen lifts.   [excerpt — source lines 74–92, 123–174]

def set_pen_pos(self, norm_x: float, norm_y: float, z_mm: float) -> None:

if not (0.0 <= norm_x <= 1.0 and 0.0 <= norm_y <= 1.0):

self._pen_last_pos = None

if self.pen_dot is not None:

self.pen_dot.setVisible(False)

return

x_px = norm_x * self.wx

y_px = (1.0 - norm_y) * self.wy

self._pen_last_pos = (x_px, y_px)

self.pen_dot.setPos(x_px, y_px)

self.pen_dot.setVisible(True)

# z_mm is the pen tip's height above the paper (mm): ~0 while drawing,

# = the lift height during pen-up travel/approach. Fade the dot when the

# pen is lifted so it only reads as a solid mark while actually drawing

# (the begin pose maps to the canvas centre, so a solid dot would

# otherwise "dip" to the middle during every move-to-begin / travel).

lifted = z_mm > 2.0

alpha = 70 if lifted else 255

self.pen_dot.setBrush(QBrush(QColor(135, 206, 235, alpha)))

# ...

def _scene_pt(self, view_pos):

"""Map a viewport point to scene mm and report whether it lies inside

the drawable workspace [0,wx]x[0,wy]. Strokes capture ONLY in-bounds

points: when the mouse leaves the canvas the stroke must HOLD, not

trail the pen along the workspace edge. Trailing the edge drives the

robot to its min/max reach where it wobbles — exactly the behaviour to

avoid. Returns (QPointF scene_mm, inside_bool)."""

p = self.mapToScene(view_pos)

inside = (0.0 <= p.x() <= self.wx) and (0.0 <= p.y() <= self.wy)

return p, inside

def mousePressEvent(self, event):

p, inside = self._scene_pt(event.pos())

if not inside:

return  # ignore presses that start outside the drawable canvas

if self._t0 is None:

self._t0 = time.time()

self.current_points = []

self.current_path = QPainterPath(p)

self.current_points.append(

{"x": p.x(), "y": p.y(), "t": time.time() - self._t0, "p": 0.5}

)

def mouseMoveEvent(self, event):

if self.current_path is None:

return

p, inside = self._scene_pt(event.pos())

if not inside:

return  # mouse left the canvas — hold the stroke, don't trail the edge

self.current_path.lineTo(p)

self.scene_.clear()

for s in self.strokes:

self.scene_.addPath(s["qpath"], self._cosmetic_pen())

self.scene_.addPath(self.current_path, self._cosmetic_pen())

self._ensure_pen_dot()  # scene_.clear() destroyed the dot's C++ object

self.current_points.append(

{"x": p.x(), "y": p.y(), "t": time.time() - self._t0, "p": 0.5}

)

def mouseReleaseEvent(self, event):

if self.current_path is None:

return

self.strokes.append({"qpath": self.current_path, "points": self.current_points})

self.current_path = None

def get_drawing(self) -> dict:

return {

"canvas": {"width": self.wx, "height": self.wy, "units": "mm"},

"strokes": [

{"id": i, "points": s["points"]} for i, s in enumerate(self.strokes)

],

}

# 2  Motion Backend Nodes

The pendant launches and drives a set of ROS 2 nodes that perform the kinematics and trajectory generation, all built directly from the latched /robot_description rather than an idealised DH model. They communicate with the pendant only through the contract topics.

## Kinematics Library — URDF Jacobian and DLS Solvers

src/arm_bot/arm_bot/ik_lib.py   ·   Shared kinematics: forward kinematics and the 6xN geometric Jacobian built from the URDF, and the interface and documented design of the task-priority pen-tip solver (position primary, pen-perpendicular orientation solved in the null space) used for tilt-capable drawing.   [excerpt — source lines 62–64, 116–143, 290–319]

class UrdfChain:

"""Serial-chain FK + geometric Jacobian built from a URDF."""

# ...

def fk(self, q: np.ndarray):

"""Return list of T_world at every joint origin (pre-rotation) and T_ee."""

T = np.eye(4)

T_at_origin = []

qi = 0

for j in self.joints:

T = T @ j["T_origin"]

T_at_origin.append(T.copy())

if j["type"] in ("revolute", "continuous"):

R = _axis_angle_R(j["axis"], q[qi])

Rh = np.eye(4)

Rh[:3, :3] = R

T = T @ Rh

qi += 1

return T_at_origin, T

def jacobian(self, q: np.ndarray):

T_at_origin, T_ee = self.fk(q)

p_ee = T_ee[:3, 3]

J = np.zeros((6, self.n))

for col, joint_idx in enumerate(self.q_joints):

T_o = T_at_origin[joint_idx]

z = T_o[:3, :3] @ self.joints[joint_idx]["axis"]

z = z / np.linalg.norm(z)

o = T_o[:3, 3]

J[:3, col] = np.cross(z, p_ee - o)

J[3:, col] = z

return J, T_ee

# ...

def solve_ik_tip(

chain: UrdfChain,

p_tip_des: np.ndarray,

R_des: np.ndarray,

tool_offset: np.ndarray,

q_seed: np.ndarray,

*,

params: Optional[dict] = None,

) -> tuple[np.ndarray, float, float, bool]:

"""Task-priority IK for a TOOL TIP offset from the chain tip.

Used for tilt-capable drawing. The pen tip is ``tool_offset`` (a vector in

the chain-tip local frame, e.g. ``[pen_len, 0, 0]``) away from the IK tip

link. Two prioritised tasks:

1. PRIMARY  — tip position ``p_tip_des`` (3-DOF). Always solved, so IK

only "fails" when the tip is genuinely outside the position-reachable

set — not because orientation couldn't be held.

2. SECONDARY — tool orientation toward ``R_des`` (pen ⟂ paper), solved in

the null space of the primary. The wrist keeps the pen perpendicular

where it has spare DOF (canvas centre) and tilts only as much as the

position task forces it to (canvas edges).

Controlling the TIP (not the chain-tip origin) is essential once tilt is

allowed: with a tilting wrist the tip swings up to ``|tool_offset|`` away

from the chain-tip origin, so commanding the origin would misplace the pen.

Returns ``(q, pos_err, tilt_rad, converged)`` where ``tilt_rad`` is the

angle between the achieved pen axis and the ``R_des`` pen axis.

"""

## Inverse-Kinematics Node — Real-Time Control Loop

src/arm_bot/arm_bot/ik_arm_v3.py   ·   The 200 Hz damped-least-squares control tick: adaptive damping, a singularity-robust damping floor from the manipulability measure, weighted DLS, and self-fading null-space joint-centering and velocity damping.   [excerpt — source lines 240–298, 306–334]

def _step(self):

if self._chain is None or self._q is None or not self._active:

return

n = self._chain.n

I6 = np.eye(6)

I3 = np.eye(3)

In = np.eye(n)

q_null = (self._null_target if self._null_target is not None

else self._chain.q_mid)

# Track a low-pass of the actual posture (updated from the measured _q,

# before the inner loop mutates it). The null-space damping below pulls

# toward this lagged posture, so it opposes fast self-motion (wobble)

# but vanishes once the arm settles (q_ref converges to q).

if self._q_ref is None:

self._q_ref = self._q.copy()

else:

self._q_ref += self._null_damp_alpha * (self._q - self._q_ref)

for _ in range(self._inner_iters):

J, T_cur = self._chain.jacobian(self._q)

p_ee = T_cur[:3, 3]

e_p = self._pos_gain * (self._T_des[:3, 3] - p_ee)

if self._position_only:

# 3-DoF task: track position only; orientation left free for the

# null-space (no wrist fight → no orientation-induced wobble).

J_task, e_task, I_task = J[:3, :], e_p, I3

err_norm = np.linalg.norm(e_p)

converged = err_norm < self._tol_pos

else:

e_r = self._rot_gain * rot_error(T_cur[:3, :3], self._T_des[:3, :3])

J_task, e_task, I_task = J, np.r_[e_p, e_r], I6

err_norm = np.sqrt(np.linalg.norm(e_p)**2 + np.linalg.norm(e_r)**2)

converged = (np.linalg.norm(e_p) < self._tol_pos and

np.linalg.norm(e_r) < self._tol_rot)

if converged:

self._active = False

break

lam = self.LAMBDA_MIN + (self.LAMBDA_MAX - self.LAMBDA_MIN) * \

min(1.0, err_norm / self.LAMBDA_KNEE)

# Weighted damped least-squares: high-weight joints move less.

W_inv = self._W_inv

JWJ = J_task @ W_inv @ J_task.T

# Singularity-robust floor: raise the damping as manipulability

# drops so the arm slows smoothly into a singularity instead of the

# pseudo-inverse amplifying a tiny Cartesian error into a large dq

# (the overshoot). Disabled when w_thresh == 0.

if self._w_thresh > 0.0:

w = np.sqrt(max(np.linalg.det(JWJ), 0.0))

engaged = w < self._w_thresh

if engaged:

ratio = 1.0 - w / self._w_thresh

lam = max(lam, self._lambda_sing * ratio * ratio)

# ...

M  = JWJ + (lam ** 2) * I_task

dq = W_inv @ J_task.T @ np.linalg.solve(M, e_task)

# Null-space (redundancy) resolution, faded out near the target so

# its leak through the damped projector can't hold a steady

# following error that stops the node deactivating (the reason jog

# used to run null_k=0). Full pull while moving, ~0 once converged.

Jp  = W_inv @ J_task.T @ np.linalg.solve(M, J_task)

N   = In - Jp                       # (damped) null-space projector

null_gain = self._null_k * min(1.0, err_norm / self.NULL_FADE)

dq += N @ (null_gain * (q_null - self._q))

# Null-space velocity damping: resist self-motion (wobble) of the

# redundant DoF by pulling toward the lagged posture; self-fading so

# it doesn't block convergence/deactivation.

if self._null_damp_k > 0.0:

dq += N @ (self._null_damp_k * (self._q_ref - self._q))

mag = np.linalg.norm(dq)

if mag > self._dq_max:

dq *= (self._dq_max / mag)

self._q = np.clip(self._q + dq, self._q_lo, self._q_hi)

cmd              = JointState()

cmd.header.stamp = self.get_clock().now().to_msg()

cmd.name         = self._names_in

cmd.position     = self._q.tolist()

self._pub.publish(cmd)

## Forward-Kinematics Node

src/arm_bot/arm_bot/fk_arm_v3.py   ·   Loads the latched /robot_description (TRANSIENT_LOCAL QoS), builds the chain, and republishes the end-effector pose on /ee_pose for every /joint_states update.   [excerpt — source lines 142–165, 180–210]

class FKNode(Node):

def __init__(self):

super().__init__("fk_7dof_v3")

self.declare_parameter("base_link", "base_link")

self.declare_parameter("tip_link",  "ee")

self._base = self.get_parameter("base_link").value

self._tip  = self.get_parameter("tip_link").value

self._chain: UrdfChain | None = None

self._joint_index = None

latched = QoSProfile(

depth=1,

durability=QoSDurabilityPolicy.TRANSIENT_LOCAL,

reliability=QoSReliabilityPolicy.RELIABLE,

)

self.create_subscription(String, "/robot_description", self._cb_urdf, latched)

self.create_subscription(JointState, "/joint_states", self._cb_joints, 30)

self._pub = self.create_publisher(PoseStamped, "/ee_pose", 30)

self.get_logger().info(

f"fk_arm_v3 ready — waiting for /robot_description (chain {self._base} → {self._tip})"

)

# ...

def _cb_joints(self, msg: JointState):

if self._chain is None:

return

if self._joint_index is None:

try:

self._joint_index = [msg.name.index(n) for n in self._chain.joint_names]

except ValueError as e:

self.get_logger().warn(

f"/joint_states missing one of {self._chain.joint_names}: {e}"

)

return

q    = np.array([msg.position[i] for i in self._joint_index], dtype=float)

T    = self._chain.fk(q)

p    = T[:3, 3]

quat = rot_to_quat(T[:3, :3])

out = PoseStamped()

out.header = msg.header

if not out.header.frame_id:

out.header.frame_id = self._base

out.pose.position.x    = float(p[0])

out.pose.position.y    = float(p[1])

out.pose.position.z    = float(p[2])

out.pose.orientation.x = float(quat[0])

out.pose.orientation.y = float(quat[1])

out.pose.orientation.z = float(quat[2])

out.pose.orientation.w = float(quat[3])

self._pub.publish(out)

## Drawing Batch Planner — Corner-Aware Waypoints

src/arm_bot/arm_bot/drawing_batch_planner.py   ·   Corner detection and per-segment resampling that keep genuine corners sharp (a single spline would round them), the corner-aware stroke splitting in the waypoint builder, and the pen-down draw loop.   [excerpt — source lines 1010–1060, 1099–1120, 1184–1199, 1239–1241]

def _detect_corners(self, xs, ys):

"""Indices of sharp-corner vertices in a polyline (mm coords).

A vertex is a corner when the turn angle (deviation from straight)

between its incoming and outgoing chord exceeds corner_angle_deg.

Sub-millimetre jitter segments are ignored so noisy freehand curves

don't register false corners. Returns a sorted list of interior

indices (empty if corner splitting is disabled or the stroke is short).

"""

n = len(xs)

if n < 3 or self.corner_angle_deg <= 0.0:

return []

thr = np.radians(self.corner_angle_deg)

min_seg = 1.0  # mm — ignore jitter shorter than this on either side

corners = []

for i in range(1, n - 1):

v1x, v1y = xs[i] - xs[i - 1], ys[i] - ys[i - 1]

v2x, v2y = xs[i + 1] - xs[i], ys[i + 1] - ys[i]

n1 = float(np.hypot(v1x, v1y))

n2 = float(np.hypot(v2x, v2y))

if n1 < min_seg or n2 < min_seg:

continue

cosang = (v1x * v2x + v1y * v2y) / (n1 * n2)

turn = float(np.arccos(np.clip(cosang, -1.0, 1.0)))

if turn > thr:

corners.append(i)

return corners

def _resample_segment(self, xs, ys):

"""Resample one polyline segment at uniform chord spacing (ds_mm).

Cubic spline for >=4 points (smooth curves), linear otherwise. Returns

(xs_s, ys_s). The original endpoints are preserved exactly, so splitting

a stroke here and stitching segments keeps corners sharp.

"""

if len(xs) < 2:

return np.asarray(xs, dtype=float), np.asarray(ys, dtype=float)

dx = np.diff(xs)

dy = np.diff(ys)

seg = np.hypot(dx, dy)

u = np.concatenate(([0.0], np.cumsum(seg)))

L = float(u[-1])

if L < 1e-6:

return np.asarray(xs, dtype=float), np.asarray(ys, dtype=float)

n_samples = max(2, int(round(L / self.ds_mm)) + 1)

u_new = np.linspace(0.0, L, n_samples)

if len(xs) >= 4:

cs_x = CubicSpline(u, xs, bc_type='natural')

cs_y = CubicSpline(u, ys, bc_type='natural')

return cs_x(u_new), cs_y(u_new)

return np.interp(u_new, u, xs), np.interp(u_new, u, ys)

# ...

corner_set: set[int] = set()

if len(xs) < 2:

xs_s, ys_s = xs.copy(), ys.copy()

else:

corners = self._detect_corners(xs, ys)

bounds = [0] + corners + [len(xs) - 1]

xparts, yparts = [], []

running = 0

for k in range(len(bounds) - 1):

a, b = bounds[k], bounds[k + 1]

rx, ry = self._resample_segment(xs[a:b + 1], ys[a:b + 1])

if k > 0:

rx, ry = rx[1:], ry[1:]   # drop the junction duplicate

xparts.append(rx)

yparts.append(ry)

running += len(rx)

# The vertex at the END of this segment (= bounds[k+1]) is a

# corner for every segment but the last.

if k < len(bounds) - 2 and len(rx) > 0:

corner_set.add(running - 1)

xs_s = np.concatenate(xparts) if xparts else xs.copy()

ys_s = np.concatenate(yparts) if yparts else ys.copy()

# ...

# 3. Draw the stroke at z_paper with constant orientation

#    First point is the touch-down (already at z_paper, q_draw).

#    Time per sample = chord_dist / v_draw.

for i in range(1, len(xs_s)):

dx = xs_s[i] - xs_s[i-1]

dy = ys_s[i] - ys_s[i-1]

d  = float(np.hypot(dx, dy))

dt = d / self.v_draw if self.v_draw > 0 else 0.0

wps.append((

float(xs_s[i]) / 1000.0,

float(ys_s[i]) / 1000.0,

self.z_paper   / 1000.0,

self.q_draw,

max(dt, 1e-3),

'draw',

))

# ...

prev_lift_xy = (xN_mm, yN_mm)

return wps

## Backend Launch Graph

src/arm_bot/launch/pendant_backend.launch.py   ·   The nodes the “Simulation ON” graph spawns and wires together — the IK node (with its soft joint limits), the FK node, and the IK-to-trajectory bridge (the drawing planner is spawned alongside).   [excerpt — source lines 139–153, 162–175]

ik_node = Node(

package="arm_bot", executable="ik_arm_v3.py", name="ik_7dof_v3",

output="screen",

parameters=[{"base_link": base_link, "tip_link": tip_link,

"use_sim_time": sim_time,

"tol_pos": 0.002,

"tol_rot": 0.01,

"inner_iters": ParameterValue(jog_inner_iters, value_type=int),

"position_only": ParameterValue(jog_position_only, value_type=bool),

"rot_gain": ParameterValue(jog_rot_gain, value_type=float),

"null_k": ParameterValue(jog_null_k, value_type=float),

"null_damp_k": ParameterValue(jog_null_damp, value_type=float),

"w_thresh": ParameterValue(jog_w_thresh, value_type=float),

"lambda_sing": ParameterValue(jog_lambda_sing, value_type=float),

"dq_max": ParameterValue(jog_dq_max, value_type=float),

# ...

"soft_q_min": [-3.14, -1.6, -3.14, -1.6, -1.6, -0.48, -1.6],

"soft_q_max": [ 3.14,  1.6,  3.14,  1.6,  1.6,  0.26,  1.6]}],

)

fk_node = Node(

package="arm_bot", executable="fk_arm_v3.py", name="fk_7dof_v3",

output="screen",

parameters=[{"base_link": base_link, "tip_link": tip_link,

"use_sim_time": sim_time}],

)

bridge = Node(

package="arm_bot", executable="ik_to_trajectory.py",

name="ik_to_trajectory", output="screen",

parameters=[{"step_horizon_s": 0.08, "use_sim_time": sim_time}],

)